SIEM HOME LAB

Complete Setup & Attack Guide

Battle-Tested — Every Problem Solved

Splunk Enterprise + Sysmon + HEC + VirtualBox

Ubuntu 24.04 LTS  |  Windows 10  |  April 2026

# Before You Start — Read This First

This guide was written after building this lab from scratch and hitting every possible problem. Every warning, fix, and note in this document is based on a real issue we encountered. Follow the steps in exact order and you will have a working SIEM lab in one session.

| Requirement | Minimum Spec |
| --- | --- |
| Host RAM | 16 GB (critical — do not attempt with 8 GB) |
| Host CPU | 4 cores (Intel or AMD x86_64 only — no ARM) |
| Host Disk | 300 GB free |
| Host OS | Windows 10/11, macOS, or Linux |
| Internet | Required for downloads only |

⚠ VirtualBox does NOT support ARM processors (Apple M1/M2/M3). If you have an Apple Silicon Mac, use UTM or Parallels instead.

# Section 1 — Download Everything First

Download all files before creating any VMs. Do not try to download while VMs are running — it significantly slows performance.

## 1.1 VirtualBox

- Go to: virtualbox.org/wiki/Downloads

- Download VirtualBox for your host OS (Windows/macOS/Linux)

- Also download the VirtualBox Extension Pack from the same page — same version number

⚠ Install the Extension Pack AFTER VirtualBox — double-click the .vbox-extpack file and click Install.

## 1.2 ISO Files

| ISO | Download From | Used For |
| --- | --- | --- |
| Ubuntu 24.04 LTS Server | ubuntu.com/download/server | SIEM VM + Linux Endpoint (same ISO) |
| Windows 10 Enterprise/Business | microsoft.com/evalcenter | Windows endpoint VM |
| Kali Linux (optional) | kali.org/get-kali | Future attacker VM |

## 1.3 Splunk Enterprise

- Go to: splunk.com/en_us/download/splunk-enterprise.html

- Create a free Splunk account (no credit card required)

- Select Linux → .deb package → Copy wget link

⚠ Do NOT sign up for Splunk Cloud. You need Splunk Enterprise (on-premises download). The wget link expires in seconds — copy it and paste immediately.

# Section 2 — VirtualBox Host-Only Network

Create this network BEFORE creating any VMs. All three VMs will use it to communicate with each other.

- Open VirtualBox → File → Host Network Manager (or Tools → Network)

- Click Create — a new adapter appears automatically

- Click the adapter → Adapter tab → Configure Adapter Manually

- Set IPv4 Address: 192.168.57.1 and Mask: 255.255.255.0

- Click DHCP Server tab → UNCHECK Enable Server

- Click Apply → Close

✓ DHCP must be disabled. If DHCP is enabled, VMs get random IPs on every boot and Splunk forwarding breaks.

Your lab network will use these static IPs:

| VM | Static IP | Role |
| --- | --- | --- |
| SIEM | 192.168.57.10 | Splunk Enterprise — receives all logs |
| WIN-ENDPOINT | 192.168.57.20 | Windows target — generates security events |
| Linux-Endpoint | 192.168.57.30 | Linux target — generates auth events |

# Section 3 — Create All Three VMs

⚠ Create ALL three VMs before starting any of them. Configure network adapters on each VM while it is powered off.

## 3.1 SIEM VM

### Create the VM

- VirtualBox → New

- Name: SIEM | Type: Linux | Version: Ubuntu 64-bit

- RAM: 4096 MB | CPUs: 2 | Disk: 100 GB (dynamically allocated VDI)

- Unattended Install: Username: labuser | Password: Lab@1234 | Hostname: SIEM | Domain: lab.local

- Uncheck Install Guest Additions (not needed for server)

### Network Adapters (Settings → Network)

- Adapter 1: NAT (leave as default)

- Adapter 2: Enable → Host-only Adapter → select VirtualBox Host-Only Ethernet Adapter

## 3.2 Windows Endpoint VM

### Create the VM

- VirtualBox → New

- Name: WIN-ENDPOINT | Type: Microsoft Windows | Version: Windows 10 64-bit

- RAM: 2048 MB | CPUs: 2 | Disk: 60 GB

- Unattended Install: Username: labuser | Password: Lab@1234 | Hostname: WIN-ENDPOINT | Domain: lab.local

- Check Install Guest Additions

⚠ Hostname must have NO spaces. WIN-ENDPOINT is correct. 'Windows 10' will cause an error.

### Network Adapters

- Adapter 1: NAT

- Adapter 2: Enable → Host-only Adapter → VirtualBox Host-Only Ethernet Adapter

## 3.3 Linux Endpoint VM

### Create the VM

- VirtualBox → New

- Name: Linux-Endpoint | Type: Linux | Version: Ubuntu 64-bit

- RAM: 512 MB | CPUs: 1 | Disk: 40 GB

- Unattended Install: Username: labuser | Password: Lab@1234 | Hostname: Linux-Endpoint | Domain: lab.local

- Uncheck Install Guest Additions

### Network Adapters

- Adapter 1: NAT

- Adapter 2: Enable → Host-only Adapter → VirtualBox Host-Only Ethernet Adapter

✓ 512 MB RAM is enough for Ubuntu Server with no GUI. This saves RAM on your host machine.

# Section 4 — Configure Ubuntu VMs (Static IP)

Do this on BOTH the SIEM VM and Linux-Endpoint VM. The only difference is the IP address. Start with the SIEM VM.

⚠ This is where most people lose hours. The most common mistake is editing the wrong netplan file or creating a file with a comma instead of a period in the filename. Follow these steps exactly.

## 4.1 Find Your Network Interfaces

After Ubuntu boots and you log in, run:

ip link show

You will see three interfaces:

- lo — loopback (ignore this)

- enp0s3 — NAT adapter (internet access)

- enp0s8 — host-only adapter (lab network — needs static IP)

⚠ If enp0s8 shows state DOWN that is normal at this point. We will bring it up after setting the IP.

## 4.2 Check Existing Netplan Files

CRITICAL — run this first before editing anything:

ls -la /etc/netplan/

You should see exactly ONE file, typically named 50-cloud-init.yaml. If you see two files, delete the extra one before proceeding.

⚠ A common mistake is creating a new file instead of editing the existing one. This creates two conflicting configs and the IP never applies. Always edit the existing file.

## 4.3 Edit the Netplan Config

sudo nano /etc/netplan/50-cloud-init.yaml

Delete everything and replace with exactly this (for SIEM VM):

network:

version: 2

ethernets:

enp0s3:

dhcp4: true

enp0s8:

dhcp4: false

addresses: [192.168.57.10/24]

For Linux-Endpoint use 192.168.57.30/24 instead.

⚠ YAML is spacing-sensitive. Use SPACES not tabs. Two spaces for each indentation level exactly as shown.

Save: Ctrl+X → Y → Enter

## 4.4 Apply the Configuration

sudo chmod 600 /etc/netplan/50-cloud-init.yaml

sudo netplan apply

sudo ip link set enp0s8 up

ip addr show enp0s8

You should see inet 192.168.57.10/24 (or .30/24 for Linux-Endpoint).

✓ If the IP does not appear, check ls -la /etc/netplan/ for duplicate files and delete any extras.

## 4.5 Install Required Packages

sudo apt update && sudo apt upgrade -y

sudo apt install -y openssh-server net-tools curl wget

## 4.6 Verify Connectivity

From SIEM VM, ping both other VMs:

ping -c 3 192.168.57.20

ping -c 3 192.168.57.30

ping -c 3 8.8.8.8

All three should reply before moving on.

# Section 5 — Install Splunk Enterprise on SIEM VM

⚠ Run all Splunk commands on the SIEM VM only. Never install Splunk on the endpoint VMs.

## 5.1 Download Splunk

Go to splunk.com, log in, go to the Splunk Enterprise download page, select Linux → .deb, and click Copy wget link. Then IMMEDIATELY paste it in the SIEM VM terminal:

wget -O splunk.deb 'https://download.splunk.com/products/splunk/releases/X.X.X/linux/splunk-X.X.X-linux-amd64.deb'

⚠ The wget link expires within seconds of copying. Have the terminal ready before clicking Copy wget link. If you get a 404 error or 0-byte file, go back and copy a fresh link immediately.

Verify the download is complete (should be ~1 GB):

ls -lh splunk.deb

## 5.2 Install Splunk

sudo dpkg -i splunk.deb

## 5.3 Fix Web Interface Binding

By default Splunk only listens on 127.0.0.1 which means you cannot access it from your host browser. Fix this BEFORE starting Splunk:

sudo mkdir -p /opt/splunk/etc/system/local

sudo bash -c 'echo "[settings]" > /opt/splunk/etc/system/local/web.conf'

sudo bash -c 'echo "server.socket_host = 0.0.0.0" >> /opt/splunk/etc/system/local/web.conf'

✓ Doing this before first start saves you from the web interface not loading problem.

## 5.4 Start Splunk

sudo /opt/splunk/bin/splunk start --accept-license --run-as-root

When prompted, create admin credentials:

- Administrator username: admin

- Password: Splunk@1234 (must be 8+ chars with mixed case and number)

Wait for: The Splunk web interface is at http://SIEM:8000

⚠ Always use --run-as-root flag. Newer Splunk versions deprecate running as root without this flag and will fail silently.

## 5.5 Enable Boot Start

sudo /opt/splunk/bin/splunk enable boot-start --run-as-root

## 5.6 Open Firewall Ports

sudo ufw allow 22/tcp

sudo ufw allow 8000/tcp

sudo ufw allow 8088/tcp

sudo ufw allow 9997/tcp

sudo ufw allow 514/tcp

sudo ufw allow 514/udp

sudo ufw enable

sudo ufw status

⚠ Open ALL ports listed above now even if you do not use them all immediately. Missing a port later causes hours of debugging.

## 5.7 Access Splunk Web UI

Open your host machine browser and go to:

http://192.168.57.10:8000

Log in with admin and the password you set. You should see the Splunk Enterprise home dashboard.

# Section 6 — Configure Splunk Indexes and Inputs

⚠ Create indexes BEFORE connecting any forwarders. If you connect forwarders first, data goes to the default index and is hard to find.

## 6.1 Create Indexes

Run on SIEM VM:

sudo /opt/splunk/bin/splunk add index wineventlog --run-as-root

sudo /opt/splunk/bin/splunk add index sysmon --run-as-root

sudo /opt/splunk/bin/splunk add index linux-logs --run-as-root

⚠ The Linux index name is linux-logs with a HYPHEN not an underscore. This caused major confusion during our build. Be consistent — always use linux-logs everywhere.

## 6.2 Enable Splunk Receiver (Port 9997)

This allows the Windows Universal Forwarder to connect:

sudo /opt/splunk/bin/splunk enable listen 9997 --run-as-root

Or via UI: Settings → Forwarding and Receiving → Configure Receiving → New Receiving Port → 9997 → Save

## 6.3 Enable HTTP Event Collector (HEC)

HEC is used by the Linux endpoint to forward logs. Set it up via the Splunk UI:

- Settings → Data Inputs → HTTP Event Collector

- Click Global Settings → Enable → Port: 8088 → Save

- Click New Token

- Name: linux-hec → Next

- Source type: syslog (NOT syslogs — no trailing s)

- Default Index: linux-logs

- Allowed Indexes: select linux-logs → move to Selected

- Review → Submit

- COPY THE TOKEN VALUE — you will need it later

⚠ The most critical step: set source type to syslog (no trailing s) and index to linux-logs. A typo here causes hours of Incorrect Index errors.

✓ When testing HEC, do NOT include the index in the JSON body. Let the token handle the index. This avoids Incorrect Index errors.

# Section 7 — Configure Windows Endpoint

## 7.1 Set Static IP

Open PowerShell as Administrator (right-click Start → Windows PowerShell Admin):

New-NetIPAddress -InterfaceAlias 'Ethernet 2' -IPAddress 192.168.57.20 -PrefixLength 24 -DefaultGateway 192.168.57.1

Set-DnsClientServerAddress -InterfaceAlias 'Ethernet 2' -ServerAddresses 8.8.8.8

Set-Service -Name SplunkForwarder -StartupType Automatic

Verify the IP:

ipconfig

Confirm 192.168.57.20 appears on Ethernet 2. Then test SIEM connectivity:

ping 192.168.57.10

⚠ If ping times out, the SIEM VM's enp0s8 interface may be down. Switch to SIEM VM and run: sudo ip link set enp0s8 up

## 7.2 Enable Security Auditing

Open Group Policy Editor (Win+R → gpedit.msc):

Navigate to: Computer Configuration → Windows Settings → Security Settings → Local Policies → Audit Policy

Enable Success AND Failure for:

- Audit logon events

- Audit account management

- Audit process tracking

- Audit policy change

## 7.3 Install Guest Additions (Clipboard)

In VirtualBox menu bar of Windows VM:

- Devices → Insert Guest Additions CD Image

- Open File Explorer inside VM → find the CD drive → run VBoxWindowsAdditions.exe

- Accept all defaults → Reboot when prompted

- After reboot: Devices → Shared Clipboard → Bidirectional

✓ This enables copy/paste between your host and Windows VM. Makes all subsequent steps much easier.

## 7.4 Install Sysmon

In PowerShell (Admin):

Invoke-WebRequest -Uri 'https://download.sysinternals.com/files/Sysmon.zip' -OutFile 'C:\Sysmon.zip'

Expand-Archive -Path 'C:\Sysmon.zip' -DestinationPath 'C:\Sysmon'

Invoke-WebRequest -Uri 'https://raw.githubusercontent.com/SwiftOnSecurity/sysmon-config/master/sysmonconfig-export.xml' -OutFile 'C:\Sysmon\sysmonconfig.xml'

C:\Sysmon\Sysmon64.exe -accepteula -i C:\Sysmon\sysmonconfig.xml

Get-Service Sysmon64

Should show Status: Running

⚠ The URL contains 'Sysmon.zip' with capital S. A typo here (Symson.zip) causes a 404 error.

## 7.5 Install Splunk Universal Forwarder

Download from splunk.com/en_us/download/universal-forwarder.html inside the Windows VM browser. Select Windows 64-bit .msi.

Run the installer:

- Check Accept License Agreement

- Select An on-premises Splunk Enterprise instance

- Click Customize Options

- Account type: Local System

- Check: Security Log, Application Log, System Log, Forwarded Events Log

- Deployment Server screen: leave BLANK (do not enter anything here)

- Receiving Indexer screen: leave BLANK

- Click Install

⚠ Do NOT enter the SIEM IP in the Deployment Server screen. Leave it blank. We configure the connection manually via config files after install.

After installation, create the config files in PowerShell (Admin):

Create inputs.conf:

$inputs = @'

[WinEventLog://Security]

disabled = 0

index = wineventlog

[WinEventLog://System]

disabled = 0

index = wineventlog

[WinEventLog://Application]

disabled = 0

index = wineventlog

[WinEventLog://Microsoft-Windows-Sysmon/Operational]

disabled = 0

index = sysmon

renderXml = true

'@

$inputs | Out-File -FilePath 'C:\Program Files\SplunkUniversalForwarder\etc\system\local\inputs.conf' -Encoding ASCII

Create outputs.conf:

$outputs = @'

[tcpout]

defaultGroup = splunk_indexer

[tcpout:splunk_indexer]

server = 192.168.57.10:9997

'@

$outputs | Out-File -FilePath 'C:\Program Files\SplunkUniversalForwarder\etc\system\local\outputs.conf' -Encoding ASCII

Restart and verify:

Restart-Service SplunkForwarder

Get-Service SplunkForwarder

Test-NetConnection -ComputerName 192.168.57.10 -Port 9997

TcpTestSucceeded: True confirms the forwarder is connected to Splunk.

✓ If SplunkForwarder shows Stopped, run: Start-Service SplunkForwarder and then Set-Service -Name SplunkForwarder -StartupType Automatic

# Section 8 — Configure Linux Endpoint Log Forwarding

The Linux endpoint uses HTTP Event Collector (HEC) to forward auth.log to Splunk. This is more reliable than rsyslog/syslog forwarding on Ubuntu 24.04.

⚠ Do NOT use rsyslog UDP/TCP forwarding on Ubuntu 24.04 with Splunk 10.x. It appears to work (packets arrive) but Splunk silently drops them. Use HEC instead.

## 8.1 Set Static IP

Follow the same netplan process as Section 4 but use 192.168.57.30/24 as the IP address.

## 8.2 Install Required Packages

sudo apt update

sudo apt install -y openssh-server sshpass curl

## 8.3 SSH from Windows VM (for easy copy/paste)

The Linux-Endpoint has no Guest Additions so clipboard doesn't work. Use SSH from the Windows VM instead:

On Windows VM PowerShell:

ssh labuser@192.168.57.30

Type yes when prompted about the fingerprint, then enter password Lab@1234

✓ All commands in the rest of this section can be pasted via Ctrl+V in this SSH session.

## 8.4 Create the Log Forwarder Script

Replace YOUR-HEC-TOKEN with the token you copied from Splunk in Section 6.3:

sudo tee /usr/local/bin/forward-logs.sh << 'SCRIPT'

#!/bin/bash

TOKEN="YOUR-HEC-TOKEN-HERE"

tail -F /var/log/auth.log | while read line; do

ESCAPED=$(echo "$line" | python3 -c "import sys,json; print(json.dumps(sys.stdin.read().strip()))")

curl -sk -X POST "https://192.168.57.10:8088/services/collector" \

-H "Authorization: Splunk $TOKEN" \

-H "Content-Type: application/json" \

-d "{\"event\": $ESCAPED, \"sourcetype\": \"syslog\"}"

done

SCRIPT

sudo chmod +x /usr/local/bin/forward-logs.sh

⚠ Do NOT include index in the JSON body. Let the HEC token handle the index. Including the index causes Incorrect Index errors.

## 8.5 Create Systemd Service

sudo tee /etc/systemd/system/log-forwarder.service << 'EOF'

[Unit]

Description=Forward auth logs to Splunk HEC

After=network.target

[Service]

ExecStart=/usr/local/bin/forward-logs.sh

Restart=always

User=root

[Install]

WantedBy=multi-user.target

EOF

sudo systemctl daemon-reload

sudo systemctl enable log-forwarder.service

sudo systemctl start log-forwarder.service

sudo systemctl status log-forwarder.service

Should show: Active: active (running)

## 8.6 Test the Connection

Send a test message (replace token with yours):

curl -sk -X POST "https://192.168.57.10:8088/services/collector" \

-H "Authorization: Splunk YOUR-HEC-TOKEN-HERE" \

-H "Content-Type: application/json" \

-d '{"event":"test from Linux-Endpoint","sourcetype":"syslog"}'

Response should be: {"text":"Success","code":0}

Then verify in Splunk:

index="linux-logs" | head 10

# Section 9 — Verify All Data is Flowing

Run these searches in Splunk Search & Reporting to confirm all pipelines are working before running attacks.

| Splunk Search | Expected Result |
| --- | --- |
| index=wineventlog ¦ head 10 | Windows Security/System/Application events |
| index=sysmon ¦ head 10 | Sysmon process creation XML events |
| index="linux-logs" ¦ head 10 | Auth.log lines from Linux-Endpoint |

⚠ If any index returns 0 events, do not proceed to attacks. Fix the pipeline first using the troubleshooting section at the end of this guide.

# Section 10 — Attack Simulations

Run each attack, then immediately search Splunk to confirm detection before saving the alert.

## Attack 1 — Windows Brute Force (MITRE T1110)

### Run on Windows VM PowerShell (Admin)

1..20 | ForEach-Object {

$cred = New-Object System.Management.Automation.PSCredential(

'fakeadmin', (ConvertTo-SecureString "wrongpassword$_" -AsPlainText -Force))

try { Start-Process cmd -Credential $cred -WindowStyle Hidden } catch {}

Start-Sleep -Milliseconds 500

}

### Detection Query

index=wineventlog EventCode=4625

| bucket _time span=5m

| stats count by _time, ComputerName

| where count > 5

| rename count as Failed_Attempts

### Save as Alert

- Name: Brute Force Detection | Cron: */5 * * * * | Severity: High

## Attack 2 — Encoded PowerShell (MITRE T1059.001)

### Run on Windows VM PowerShell (Admin)

powershell -EncodedCommand "V3JpdGUtSG9zdCAnU3VzcGljaW91cyBQb3dlclNoZWxsIERldGVjdGVkISc="

### Detection Query

index=sysmon source="WinEventLog:Microsoft-Windows-Sysmon/Operational"

| search _raw="*EncodedCommand*"

| rex field=_raw "CommandLine>(?P<CommandLine>[^&]+)"

| rex field=_raw "User>(?P<User>[^&]+)"

| table _time, host, User, CommandLine

### Save as Alert

- Name: Suspicious Encoded PowerShell | Cron: */5 * * * * | Severity: High

## Attack 3 — New Admin Account (MITRE T1136.001)

### Run on Windows VM PowerShell (Admin)

net user hacker Passw0rd123! /add

net localgroup administrators hacker /add

### Detection Query

index=wineventlog EventCode=4720 OR EventCode=4732

| table _time, ComputerName, Account_Name

### Cleanup after testing

net user hacker /delete

### Save as Alert

- Name: New Admin Account Created | Cron: */5 * * * * | Severity: Critical

## Attack 4 — Linux SSH Brute Force (MITRE T1110)

### Run from SIEM VM (attacking Linux-Endpoint)

sudo apt install -y sshpass

for i in {1..20}; do sshpass -p "wrongpassword" ssh -o StrictHostKeyChecking=no baduser@192.168.57.30 2>/dev/null; done

### Also run from Linux-Endpoint (attacking itself)

for i in {1..15}; do sshpass -p "wrongpassword" ssh -o StrictHostKeyChecking=no baduser@localhost 2>/dev/null; done

### Detection Query

index="linux-logs" "Failed password"

| rex field=_raw "Failed password for (?:invalid user )?(?P<username>\w+) from (?P<src_ip>[\w:\.]+)"

| stats count by src_ip, username

| where count > 5

### Save as Alert

- Name: Linux SSH Brute Force | Cron: */5 * * * * | Severity: High

# Section 11 — Build the SOC Dashboard

In Splunk: Dashboards → Create New Dashboard → Title: SOC Monitoring Dashboard → Classic Dashboard → Create

| Panel Title | SPL Query | Type |
| --- | --- | --- |
| Failed Logons Over Time | index=wineventlog EventCode=4625 ¦ timechart span=5m count as Failed_Logons | Line Chart |
| Top Targeted Accounts | index=wineventlog EventCode=4625 ¦ stats count by Account_Name ¦ sort -count ¦ head 10 | Bar Chart |
| Encoded PowerShell | index=sysmon _raw=*EncodedCommand* ¦ table _time, host, _raw | Events Viewer |
| New Admin Accounts | index=wineventlog EventCode=4720 OR EventCode=4732 ¦ table _time, ComputerName, Account_Name | Events Viewer |
| Total Windows Events | index=wineventlog ¦ stats count as Total_Events | Single Value |
| Top Event IDs | index=wineventlog ¦ stats count by EventCode ¦ sort -count ¦ head 10 | Pie Chart |
| SSH Attack Sources | index="linux-logs" "Failed password" ¦ rex field=_raw "from (?P<src_ip>[\w:\.]+) port" ¦ stats count by src_ip ¦ sort -count | Bar Chart |
| SSH Failures Over Time | index="linux-logs" "Failed password" ¦ timechart span=5m count as SSH_Failures | Line Chart |
| Total Linux Events | index="linux-logs" ¦ stats count as Linux_Events | Single Value |

Click Save after adding all panels.

# Section 12 — Key Event ID Reference

| Source | Event ID | Description | Detects |
| --- | --- | --- | --- |
| Windows | 4625 | Failed logon | Brute force / password spray |
| Windows | 4624 | Successful logon | Lateral movement after brute force |
| Windows | 4720 | User account created | Backdoor account creation |
| Windows | 4732 | Member added to group | Privilege escalation |
| Windows | 4698 | Scheduled task created | Persistence mechanism |
| Sysmon | 1 | Process creation | Malware / encoded PowerShell |
| Sysmon | 3 | Network connection | C2 communication |
| Sysmon | 11 | File created | Malware dropper |
| Sysmon | 22 | DNS query | C2 domain resolution |
| Linux | auth.log | SSH Failed password | SSH brute force |

# Section 13 — Troubleshooting Guide

Every problem listed here was encountered and solved during the actual build of this lab.

## 13.1 Splunk Issues

### Splunk web UI not accessible from host browser

Cause: Splunk binds to 127.0.0.1 by default. Fix:

sudo bash -c 'echo "[settings]" > /opt/splunk/etc/system/local/web.conf'

sudo bash -c 'echo "server.socket_host = 0.0.0.0" >> /opt/splunk/etc/system/local/web.conf'

sudo /opt/splunk/bin/splunk restart --run-as-root

### Splunk start fails with root deprecation warning

Always use the --run-as-root flag:

sudo /opt/splunk/bin/splunk start --accept-license --run-as-root

### Splunk shows 0 events after forwarder connects

Cause: Index does not exist or name mismatch. Fix — create indexes first:

sudo /opt/splunk/bin/splunk add index wineventlog --run-as-root

sudo /opt/splunk/bin/splunk add index sysmon --run-as-root

sudo /opt/splunk/bin/splunk add index linux-logs --run-as-root

### SplunkForwarder service stops immediately

Fix:

Start-Service SplunkForwarder

Set-Service -Name SplunkForwarder -StartupType Automatic

## 13.2 Network Issues

### Ubuntu VM shows IP as DOWN or no IP on enp0s8

Fix:

sudo ip link set enp0s8 up

sudo netplan apply

### Netplan config not applying after reboot

Cause: Duplicate netplan files. Check:

ls -la /etc/netplan/

If you see two files, delete the wrong one (the one with a comma or extra characters in the name):

sudo rm /etc/netplan/WRONGFILENAME

sudo netplan apply

### ping works but SSH connection refused

SSH server not installed. Fix:

sudo apt install -y openssh-server

sudo systemctl enable ssh && sudo systemctl start ssh

## 13.3 Linux Log Forwarding Issues

### HEC returns Incorrect index error

Causes and fixes:

- Source type typo: syslogs vs syslog — must be syslog without trailing s

- Index specified in JSON body — remove the index field from the JSON, let the token handle it

- Token not linked to linux-logs — go to Settings → Data Inputs → HTTP Event Collector → click token → verify Default Index is linux-logs

### Log forwarder service shows inactive/dead

Fix:

sudo systemctl start log-forwarder.service

sudo systemctl enable log-forwarder.service

sudo journalctl -u log-forwarder.service -n 20

### tcpdump shows packets arriving but Splunk shows nothing

Cause: rsyslog UDP/TCP forwarding on Ubuntu 24.04 with Splunk 10.x has a known compatibility issue. Use HEC instead (Section 8).

## 13.4 Performance Issues

### Host machine lags with all VMs running

Reduce VM RAM allocations:

- SIEM: reduce from 6144 MB to 4096 MB

- Linux-Endpoint: reduce from 2048 MB to 512 MB

- Windows: keep at 2048 MB minimum

Change in VirtualBox: select VM (powered off) → Settings → System → Base Memory

### Splunk does not start after VM reboot

Start manually each session:

sudo /opt/splunk/bin/splunk start --accept-license --run-as-root

# Section 14 — Daily Lab Startup Checklist

Follow this order every time you start a lab session:

## Start Order

- Start SIEM VM first → wait 60 seconds

- Start Splunk: sudo /opt/splunk/bin/splunk start --accept-license --run-as-root

- Verify SIEM IP: ip addr show enp0s8 (should show 192.168.57.10)

- If IP missing: sudo ip link set enp0s8 up && sudo netplan apply

- Start Windows VM → wait for desktop

- Start Linux-Endpoint VM

- Verify Linux-Endpoint IP: ip addr show enp0s8 (should show 192.168.57.30)

- If IP missing: sudo ip link set enp0s8 up

- Start log forwarder: sudo systemctl start log-forwarder.service

- Open host browser → http://192.168.57.10:8000 → verify login works

- Run quick verification: index=wineventlog | head 5

## Save Order (End of Session)

- Save state on Linux-Endpoint first

- Save state on Windows VM

- Save state on SIEM VM last

✓ Use Save State instead of Power Off. This resumes in seconds and keeps Splunk running in memory.

| Quick Reference Command | Purpose |
| --- | --- |
| sudo /opt/splunk/bin/splunk start --accept-license --run-as-root | Start Splunk |
| sudo /opt/splunk/bin/splunk status --run-as-root | Check Splunk status |
| sudo /opt/splunk/bin/splunk restart --run-as-root | Restart Splunk |
| sudo ip link set enp0s8 up | Bring up lab network interface |
| sudo systemctl start log-forwarder.service | Start Linux log forwarding |
| sudo systemctl status log-forwarder.service | Check log forwarder |
| Get-Service SplunkForwarder | Check Windows forwarder (PowerShell) |
| Restart-Service SplunkForwarder | Restart Windows forwarder |

SIEM Home Lab — Complete Setup Guide | April 2026

Splunk Enterprise 10.2.2  +  Sysmon v15.20  +  HEC  +  VirtualBox 7  +  Ubuntu 24.04  +  Windows 10